Fare Collection Validator – AUVAL04

- High Definition 7 Inch display

- Android Operating System

- Accepts Banking Cards

- Accepts RFID Cards / NFC Phone

- QR Code Reader

- Emergency Control

- Remote Device and Content Management

Product Datasheet – AUVAL Series

Connectivity Options

Connectivity over LAN, Wi-Fi or Bluetooth.

Separate secure communication channel for sensitive card data

Security Options

EMV certified hardware with support for Mastercard, Visa, RuPay, Amex and others

Integration Options

Designed to integrate with any Turnstile hardware designs and multiple AFCS system backend on standard integration protocols.

The validator comes with various customization options of size, material and form factor. Aumnatic shall provide complete support during software development and post deployment support.

AUVAL04 is Automatic Gate or Access control Turnstile Validator which can accept various forms of validation like QR Code (Paper or  Mobile), Smart Card, Smart Wearables, Smart Phones and Banking Cards.

It is suitable for Automatic Fare Collection System as well as building access control systems. The smart mechanical design helps the support executive achieve a very aggressive MTTR (Mean Time To Recovery) of less than 10 minutes. The special snap-in and snap-out feature helps the support executive to meet the Service Levels easily.

Product Datasheet – AUVAL04

Dimension:  375 X 205 X 65 (mm) – customization available

Material – MS / SS / ABS options

Validation Media: QR Code; Mastercard, Visa, Rupay, Mifare, Desfire

Mounting Provisions: Customized options available

Operating Temperature: -20C to 60C

Storage Temperature: -40°C to 70°C

Humidity: 5% to 95%, non-condensing

Operating System: Android 11 and above and Linux

SAM Options: Upto 4 SAM

Communication: Ethernet, WI-Fi, Bluetooth, RS232

Active Screen Size: 7 inch Diagonal with Capacitive Touch Panel

QR Code Reader: 752X480 CMOS with 38.3mm X 60.4mm scan window

Detailed specification of each component can be shared on request. Various customization options are available based on the specific requirement and size of the Turnstile top cover and installation provisions.

Installation Examples

Specifications

SUPPORT AVAILABLE

Technical Support

Installation and Setup

Maintenance

Application Support

Hardware Support

Integration Support

Software Partners